RTL-to-GDS Flow: Timing-Related Report

ALU Design – Cadence Genus / Innovus

Purpose: This report marks the timing-related stages, commands, reports, warnings, and results visible in the supplied Innovus log. The supplied log provides direct evidence mainly for the physical-design, extraction, post-route timing, and signoff stages.

# 1. RTL-to-GDS Timing Flow

| Stage | Timing relevance | Objective |
| --- | --- | --- |
| RTL | Functional RTL description of the ALU. | Define correct synchronous functionality. |
| Constraints / SDC | Defines clock, input/output delays, transition, load, uncertainty and timing exceptions. | Tell synthesis and STA what timing must be met. |
| Logic Synthesis – Genus | Maps RTL to standard cells and performs timing-driven logic optimization. | Meet timing while controlling area/power. |
| Floorplanning / Placement – Innovus | Physical locations and wirelength affect cell/net delay. | Create a legal, routable physical implementation. |
| CTS | Clock tree insertion delay, skew and clock transition affect setup/hold. | Build a balanced clock distribution network. |
| Routing | Signal and clock interconnect introduce resistance/capacitance. | Create physical connections while meeting DRC/routing requirements. |
| Post-route RC Extraction | Extracts parasitic R and C used for delay calculation. | Obtain realistic post-route delays. |
| Post-route STA | Checks setup and hold using MMMC views and extracted parasitics. | Verify timing closure. |
| Timing Optimization | Uses physical optimization to repair setup/hold violations. | Remove negative slack. |
| GDSII | Streams the final physical database to GDSII. | Produce the layout database for downstream signoff/fabrication. |

# 2. Timing-Related Items to Mark

Clock definition and clock period/frequency

Input delay and output delay

Clock uncertainty

Clock transition / slew

Input transition

Output load

Timing libraries and PVT/operating corners

MMMC setup and hold views

Setup slack: WNS and TNS

Hold slack: WNS and TNS

Number of violating paths

Cell delay and net delay

Clock insertion delay and skew

Parasitic resistance and capacitance

SI/coupling-aware timing

Timing optimization

Max transition / max capacitance / max fanout constraints

# 3. Timing Results in the Supplied Innovus Log

Setup timing: WorstCase setup reports WNS = +0.990 ns, TNS = 0.000 ns, and 0 violating paths. Reg-to-reg setup WNS is +2.550 ns.

Hold timing: The first reported BestCase hold summary gives WNS = +0.032 ns, TNS = 0.000 ns, and 0 violating paths. The later SI-aware post-route calculation gives WNS = +0.048 ns, TNS = 0.000 ns, and 0 violating paths.

DRVs: The setup timing summary reports zero max-capacitance, max-transition, max-fanout and max-length violations.

# 4. Timing Results Table

| Stage / View | Check | Corner/View | WNS (ns) | TNS (ns) | Violations |
| --- | --- | --- | --- | --- | --- |
| Post-route timeDesign | Setup | WorstCase | +0.990 | 0.000 | 0 |
| Post-route timeDesign | Setup reg2reg | WorstCase | +2.550 | 0.000 | 0 |
| Post-route timeDesign | Hold | BestCase | +0.032 | 0.000 | 0 |
| Later SI-aware post-route | Hold | BestCase | +0.048 | 0.000 | 0 |

# 5. Parasitics and Timing Accuracy

The log shows post-route RC extraction followed by delay calculation. In the later extraction, 2,485 resistors, 2,439 ground capacitances and 3,004 coupling capacitances were extracted.

IMPEXT-3530: the process node is not set; Innovus recommends setting the process node before extraction for maximum accuracy.

IMPEXT-3493: extraction status can be reset after changes to analysis views, RC corners or extraction modes; re-extraction or SPEF/RCDB loading may then be required.

The later timing calculation uses SPEF/RCDB parasitics and SI-aware timing.

# 6. SI / Crosstalk-Aware Timing

The later post-route calculation shows SI iteration 1 and iteration 2, followed by glitch analysis and timing-graph rebuilding. This is the more detailed post-route timing stage in the supplied log.

The first SI iteration selected about 98.1% of the 155 nets for SI analysis.

The second SI iteration performed additional glitch analysis before the final timing summary.

# 7. Timing Optimization

For a post-route hold violation, the relevant Innovus optimization command is:

optDesign -postRoute -hold

Then recheck the result with:

timeDesign -postRoute -hold

# 8. Timing Commands to Include in an RTL-to-GDS Report

| Tool | Command | Purpose |
| --- | --- | --- |
| Genus | read_sdc | Read timing constraints. |
| Genus | syn_generic / syn_map / syn_opt | Timing-driven synthesis optimization. |
| Innovus | timeDesign -preCTS -setup | Setup check before CTS. |
| Innovus | timeDesign -postCTS -setup | Setup check after CTS. |
| Innovus | timeDesign -postCTS -hold | Hold check after CTS. |
| Innovus | extractRC | Extract post-route parasitics. |
| Innovus | timeDesign -postRoute -setup | Final/post-route setup check. |
| Innovus | timeDesign -postRoute -hold | Final/post-route hold check. |
| Innovus | report_timing -delay_type min -max_paths 10 | Inspect minimum-delay/hold paths. |
| Innovus | optDesign -postRoute -hold | Post-route hold optimization. |
| Innovus | optDesign -postRoute -setup | Post-route setup optimization. |

# 9. Timing Signoff Checklist

☐ Clock correctly defined in SDC.

☐ Input/output delays constrained.

☐ Clock uncertainty included where required.

☐ Correct timing libraries loaded for required corners.

☐ MMMC setup and hold views defined.

☐ Setup WNS >= 0 ns.

☐ Setup TNS = 0 ns.

☐ Setup violating paths = 0.

☐ Hold WNS >= 0 ns.

☐ Hold TNS = 0 ns.

☐ Hold violating paths = 0.

☐ No max-transition violations.

☐ No max-capacitance violations.

☐ No max-fanout violations.

☐ Post-route RC extraction completed.

☐ Final timing checked with extracted parasitics.

☐ SI-aware timing checked when required.

☐ Timing optimization followed by a fresh timing report.

# 10. Other Physical Issues in the Supplied Log

IMPSP-372: mismatched FollowPin in rows. This is a physical implementation issue, not a setup/hold slack number.

Verify DRC reports one METAL1 spacing violation.

Connectivity reports IO pins as not assigned in warning messages, although the final connectivity summary reports 0 violations and 0 warnings.

Routing overflow is 0.00% horizontal and 0.00% vertical.

Antenna verification reports 0 violations.

GDSII stream-out completed.

# 11. Final Timing Status

SETUP: PASS — WorstCase WNS = +0.990 ns, TNS = 0, 0 violating paths.

HOLD: PASS — BestCase WNS = +0.032 ns in the first hold summary and +0.048 ns in the later SI-aware post-route summary; TNS = 0 and 0 violating paths.

TIMING ACCURACY NOTE: The log contains a process-node extraction warning and an IMPSP-372 physical warning. These are not negative setup/hold slack, but they should be addressed before claiming the entire physical flow is clean.

# 12. Source Evidence

The numerical timing results and signoff observations in this report are taken from the supplied Innovus log: setup summary at lines 20–29, first hold summary at lines 81–90, later SI-aware hold summary at lines 180–189, and extraction/follow-pin messages at lines 101–141.